Date: March 30, 2026

1. Fixed network protocol stack memory issues.

2. Optimized network protocol stack underlying drivers.

3. Enabled DHCP dynamic IP address acquisition.

4. Fixed network audio and USB sound card audio source selection logic.

5. Fixed high VSWR protection issues in the UV band.

6. Fixed power consistency issues between handheld microphone PTT and button PTT.

7. Fixed knob lag issues.

8. Fixed compatibility issues with older GPS modules.

9. Fixed abnormal high VSWR protection in NFM mode under the 21MHz band.

Date: November 21, 2025

1. Fixed issues with discontinuous calls and occasional crashes when frequency hopping is enabled; frequency hopping will disable spectrum and waterfall charts to avoid affecting frequency hopping interruptions.

2. Added product signal setting protocol commands to enhance product control capabilities.

3. Fixed an issue where entering the correct IP address occasionally resulted in a false error when modifying the IP address.

4. Fixed an issue where the device could not boot after an upgrade due to a FLASH ECC error.

5. Fixed the issue of the programming software failing to initialize channels; frequency limiting is now enabled by default with OUT_BAND_EN enabled.

6. Fixed the issue of UI freezing when adjusting PA during NFM transmission.

7. Fixed the issue of the last option on the BAND page going out of bounds.

8. Memory management optimization.

Date: 2025.06.24

1. Added adjustable power, but the adjustment precision is insufficient.

2. Modified the VSWR calculation algorithm for more accurate standing wave calculation.

3. Disabled the network port for faster boot-up.

4. Added ALE function; this power level enables automatic radio networking.

5. Due to the addition of ALE function, voltage display has been removed. A future update will remove ALE function and display voltage.

6. Customized the GUOHETEC WSJTX software, which directly supports GUOHETEC radio product protocols.

7. Customized a HAMLIB-based dynamic library; software based on hamlib can support GUOHETEC products by replacing the dynamic library.

8. Resolved the black screen issue after firmware refresh.

Date: 2024.11.16

1. Added a Bluetooth microphone control interface.

2. Completed the function of controlling the radio via Ethernet TCP and sending/receiving audio data via UDP.

3. Added transmission in the 300~400MHz frequency band, supporting the international MARS satellite system.

4. Optimized the encoder algorithm for more precise adjustments.

5. Added a new GPS module driver.

6. Added GPS data pass-through, allowing synchronization of computer time using GPS data.

7. Increased the number of channels from 100 to 1000.

8. Increased the frequency hopping count from 30 hops per second to 1~1000 hops per second.

9. Added Ethernet read/write channels.

Date: 2024.7.13

1. Resolved the issue of not being able to connect to a power amplifier when the instantaneous transmission power exceeds 5 watts, but further algorithm optimization is needed.

2. Added network port driver, enabling data transmission and reception and radio control via the network port. However, this firmware release disables this function, as the host computer software is not yet complete.

Date: 2024.1.9

- Fixed a bug where some devices could not read EEPROM data.

Date: 2023.12.27

- Fixed a bug related to antenna tuning restart.

Date: 2023.12.19

1. Fixed a black screen issue upon startup.

2. Further stabilized DMR repeater functionality; tested repeater was Hytera RD980.

3. Added adjustable power, but the adjustment speed is not yet at the ideal level of fast and precise control, requiring further algorithm improvements. This function was actually added in the first release. The 144MHz band NFM mode cannot be changed due to hardware limitations.

4. Fixed a bug where the approval code was displayed incorrectly.

Date: 2023.12.05

1. Added network port driver; further development of the protocol layer is needed.

2. Improved the issue of interference with key presses during transmission.

3. Fixed a bug where the encoder could not correctly control parameters.

4. Fixed a bug where the input method window could not be exited.

5. Fixed the issue of DMR modification preventing relay connection; it is currently not very stable and requires further improvement.

6. Added a high-power mode for the UV band, but it is recommended to use the low-power mode as much as possible. The high-power mode may damage the radio due to the lack of calibration parameters in early shipments.

7. Fixed the issue of not being able to receive after pressing PTT in radio mode; transmission is now disabled in radio mode.

8. Added the ability to modify the transmission frequency during transmission.

9. Removed the algorithm for continuous antenna tuning.

Date: 2023.4.16 FW-V3.4.0

1. Fixed the issue of insufficient frequency offset in NFM mode.

Date: 2023.3.20 FW-V3.3.2

1. Fixed the incorrect DMR call power-on mode.

2. Added MPU6050 driver.

3. Fixed the issue of channel not being saved.

4. Added channel writing function.

5. Added shortcut operation buttons for DMR settings menu.

6. Supported direct frequency operation and repeater operation in DMR mode.

7. Improved compatibility with Hytera and Motorola walkie-talkies.

Date: 2022.8.22

FW-V3.1.8

1. Resolved the issue of insufficient modulation in NFM mode.

2. Added mouse driver, allowing the mouse to function as an encoder.

3. Added GPS data output via serial port.

4. Resolved the issue of incorrect channel storage data.

Date: 2022.8.22

FW-V3.1.4

1. Resolved the issue of insufficient modulation in NFM mode.

2. Added AFO interface to output different audio in different modes, facilitating connection to external speakers.

Date: June 7, 2022

- Switching Bluetooth input to headphone audio allows you to play FT8 with the speaker off without interference.

Date: May 4, 2022

FW-V3.1.1

1. Resolved the transmit frequency offset issue.

2. Opened the user calibration interface, providing frequency, voltage, and S-meter calibration.

3. Resolved the issue of no sub-audio transmission in NFM mode.

4. Added support for the new local oscillator driver.